T-Config功能操作说明20250113

- Multi-scale Management

- Connect via serial port

- 1. Connect the scale to the PC device with a serial port cable;

- 2. Enter the correct serial port information;

- 3. Click Connect;

- 4. If the connection is successful, the left status bar will display Online.

- 5. Click Close Port to disconnect the serial port connection; Click Open Port to open the serial port connection;

- 6. If you want to change the scale name, click Rename to enter it and click Confirm.

- Connect via Ethernet/Wi-Fi

- 1. Click [+] at the top of the left status bar;

- 2. Enter the IPv4 (IP Address) and Port (Port Number);

- 3. Click Confirm to complete the connection.

- (1) Get IP Address

- Wi-Fi: Click Wi-Fi Setting on the main interface, select the Wi-Fi you want to connect to in the left column, enter the password and click Connect. If the connection is successful, the IP Address will be displayed.

- Ethernet: After connecting the network cable, enter the menu F3-3 COM3 SHOW on the scale (F3-3 DHCP set to ON) to get the IP Address.

- (2) Get Port Number

- Wi-Fi: Enter the menu F3-2 COM2 MODE on the scale, press the TARE key three times to view the port number.

- Ethernet: Enter the menu F3-3 COM3 PORT on the scale, press the TARE key once to view the port number.

- Note:

- To use Wi-Fi connection, you need to turn on the scale's Wi-Fi function and connect the device running the software to the target Wi-Fi as well.

- To use Ethernet connection, you need to turn on the network port function of the scale and connect the device running the software to the target Wi-Fi as well.

- Device Time

- 1. Click [Set Date/Time].

- 2. Manually set or synchronize the date and time of the PC device.

- Wi-Fi Setting

- 1. Select the Wi-Fi you want to connect to from the left menu bar;

- 2. Enter the password and click [Connect] to obtain the IP address;

- 3. Return to the main interface and enter [Multi-scale Management].

- 4. Click [+] at the top of the left status bar, enter the IP address and serial port number;

- 5. Click [Confirm] to connect.

- Update Firmware

- 1. Click [Select Firmware Zip] to upload the file(.zip);

- 2. Click [Download];

- 3. Select the way to update the firmware: Serial Port or Network(Wi-Fi, Ethernet).

- Note:

- Update via serial port , the scale will automatically restart;

- Update via Wi-Fi or Ethernet, the scale will not automatically restart. You need to manually restart the scale, and the firmware will update. (This method supports selecting multi-scales to update together.)

- Label Format Download

- 1. Follow the prompts to select the corresponding print format for uploading;

- 2. Click [Download] to download the format(.fmt);

- 3. If multiple networks are connected, you need to select the corresponding network before downloading.

- Receipt Format Download

- 1. Follow the prompts to select the corresponding print format for uploading;

- 2. Click [Download] to download the format(.fmt);

- 3. If multiple networks are connected, you need to select the corresponding network before downloading.

- Label Design

- 1. Select printer protocol, length and width of the ticket, printing direction and other information;

- 2. When editing the print format, there are 4 variables to choose from in the left menu bar: Free Text, BarCode Variable, QrCode Variable, Variable;

- 3. BarCode Edit: Click [BarCode Edit] and then Click [Add]; Select format information such as data type, content, length, etc; Enter the format name and click [Save]; You can select the edited barcode format from the left menu bar.

- 4. Qrcode Edit: Same as Barcode.

- 5. After editing the format, click [Save Format] to save the print format.

- Receipt Design

- 1. Select printer protocol, length and width of the ticket, printing direction and other information;

- 2. When editing the print format, there are 4 variables to choose from in the left menu bar;

- Free Text: Text variables that can be freely inputted

- Dividing Line: A horizontal line used for segmentation (after selecting this variable twice, the content in the middle of the variable can appear repeatedly, such as the accumulated content each time)

- Variable: Variables required for weighing scales

- Price Variable: Variables required for the price computing scale

- 3. After editing the format, click [Save Format] to save the print format.

- Note: The weighing scale and pricing scale cannot use the wrong variable, otherwise the printed output will be empty.

- Serial Output Design

- 1. The top menu bar has modes such as weighing, counting, price computing, etc, which can be edited by selecting different modes based on the connected scale;

- 2. The left menu bar is for variables, which can be selected according to your needs;

- 3. You can click [Open Review] to preview the output results;

- 4. After editing the format, click [Download]. Once successful, use the serial port tool to test it.

- Note: This function can only be used in serial port connection mode.

- Basic Data

- 1. Click [Get Basic Data];

- 2. T-Config will automatically collect basic data from the scale and display it synchronously on the interface.

- Parameter Setting

- 1. There are 5 parameter settings: Weighing, Factory, Serial Port, Price, Other;

- 2. The editable parameters can be set in Value column based on the content in Description column;

- 3. After setting all parameters, click the button in the bottom right corner to submit.

- 多台秤管理

- 一、通过串口连接

- 1.通过串口线将设备与电脑连接；

- 2.输入正确的串口参数和信息，点击【连接】；

- 3.如果连接成功，左侧状态栏将显示设备在线。

- 5.点击【关闭串口】，断开串口连接；点击【打开串口】打开串口连接；

- 6.如果要更改秤名，请单击【重命名】，输入完后点击【确认】。

- 二、通过网口/Wi-Fi连接

- 1.点击左侧状态栏顶部的[+]按钮；

- 2.输入IPv4（IP地址）和Port（端口号）；

- 3.点击【确认】完成连接。

- 获取IP地址

- Wi-Fi：在主界面点击【Wi-Fi设置】，在左侧栏中选择要连接的Wi-Fi，输入密码，点击【连接】。如果连接成功，会显示IP地址。

- 网口：连接网线后，可以在秤的菜单中查看到IP地址。（菜单位置：F3-3 COM3 SHOW，其中F3-3 DHCP要设置为ON）

- 获取端口号

- Wi-Fi：进入秤的菜单F3-2 COM2 MODE，按TARE键三次查看端口号。

- 网口：进入秤的菜单F3-3 COM3 PORT，按一次TARE键查看端口号。

- 注：

- 如果要使用Wi-Fi连接T-Config，您需要先打开秤的Wi-Fi功能，并将运行T-Config的设备连接上该Wi-Fi；如果要使用网口连接T-Config，您需要先打开秤的网口功能，并将运行T-Config的设备连接到该网络。

- 设备时间

- 1.点击【设置日期/时间】可以手动设置秤上的日期和时间；

- 2.点击【同步PC时间】会自动同步电脑设备上的日期时间。

- Wi-Fi设置

- 1.从左侧菜单栏中选择要连接的Wi-Fi；

- 2.输入密码，点击【连接】获取IP地址（IPv4）；

- 3.返回主界面，点击【多台秤管理】；

- 4.点击左侧的[+]按钮，输入IP地址和串口号；

- 5.点击【确认】连接秤。

- 更新软件

- 1.点击【请选择更新文件Zip】并上传文件；

- 2.点击【下发】；

- 3. 选择固件更新方式：串口或网络（Wi-Fi、网口）；

- 注：

- 通过串口下发，秤会自动重启；

- 通过Wi-Fi或网口下发，可以批量选择多台秤，下发后秤不会自动重启，需要手动重启后才会更新软件。

- 下发标签打印格式

- 1.根据提示选择相应的标签打印格式并上传；

- 2.点击【下发】；

- 3.如果连接了多个网络，则需要在下发前选择设备对应的网络。

- 下发票据打印格式

- 1.根据提示选择相应的票据打印格式并上传；

- 2.点击【下发】；

- 3.如果连接了多个网络，则需要在下发前选择设备对应的网络。

- 标签设计

- 1.设置打印机协议、标签长度/宽度、打印方向等信息；

- 2.编辑打印格式时，左侧菜单栏中有4个变量可供选择：自由文本、条形码变量、二维码变量、变量；

- 3.条码编辑：点击【条码编辑】，点击【添加】，选择数据类型、内容、长度等格式信息；输入条码格式名称后，点击【保存】按钮后，可以在左侧菜单栏中选择已保存的条形码格式。

- 4.二维码编辑：与条码编辑相同。

- 5.编辑好标签打印格式后，点击【保存格式】。

- 票据设计

- 1.选择打印机协议、票据长度/宽度、打印方向等信息；

- 2.编辑打印格式时，左侧菜单栏中有4个变量可供选择；

- 自由文本：可以自由输入的文本变量

- 分割线：用于分割的水平线（选择该变量两次后，变量中间的内容可以重复出现，比如每次累积的内容）

- 变量：计重秤所需的变量

- 价格变量：计价秤所需的变量

- 3.编辑好票据打印格式后，点击【保存格式】。

- 注：计重秤和计价秤不能使用错误的变量，否则打印出来是空的。

- 串口输出设计

- 1.顶部菜单栏有计重、计数、计价等模式，可以根据连接的秤选择不同的模式进行设置；

- 2.左侧菜单栏为变量，可根据需要选择；

- 3.点击【打开预览】可以预览输出结果；

- 4.编辑好串口输出格式后，点击【下发】。成功后，使用串口工具进行测试。

- 基本数据

- 1.点击【获取基本数据】；

- 2.T-Config会自动收集秤上的基本数据，并在界面上同步显示。

- 参数设置

- 1.一共有5个参数设置：称重信息、出厂参数、串口参数、价格参数、其他；

- 2.“可编辑”列中显示“是”的参数，可以根据“描述”列中的提示内容在“值”列中进行编辑；

- 3.设置完所有参数后，点击右下角的按钮确认提交。

- 注：此功能只能在串口连接模式下使用。