- T-Industry功能操作说明20250113

- Multi-scale Management

- Connect via serial port

- 1. Connect the scale to the PC device with a serial port cable;

- 2. Enter the correct serial port information;

- 3. Click [Connect];

- 4. If the connection is successful, the left status bar will display Online.

- 5. Click [Close Port] to disconnect the serial port connection; Click [Open Port] to open the serial port connection;

- 6. If you want to change the scale name, click [Rename] to enter it and click [Confirm].

- Connect via Ethernet/Wi-Fi

- 1. Click [+] at the top of the left status bar;

- 2. Enter the IPv4 (IP Address) and Port (Port Number);

- 3. Click [Confirm] to complete the connection.

- (1) Get IP Address

- Wi-Fi: Click [Wi-Fi Setting] on the main interface, select the Wi-Fi you want to connect to in the left column, enter the password and click [Connect]. If the connection is successful, the IP Address will be displayed.

- Ethernet: After connecting the network cable, enter the menu F3-3 COM3 SHOW on the scale (F3-3 DHCP set to ON) to get the IP Address.

- (2) Get Port Number

- Wi-Fi: Enter the menu F3-2 COM2 MODE on the scale, press the TARE key three times to view the port number.

- Ethernet: Enter the menu F3-3 COM3 PORT on the scale, press the TARE key once to view the port number.

- Note:

- To use Wi-Fi connection, you need to turn on the scale's Wi-Fi function and connect the device running the software to the target Wi-Fi as well.

- To use Ethernet connection, you need to turn on the network port function of the scale and connect the device running the software to the target Wi-Fi as well.

- Wi-Fi Setting

- 1. Select the Wi-Fi you want to connect to from the left menu bar;

- 2. Enter the password and click [Connect] to obtain the IP address;

- 3. Return to the main interface and enter [Multi-scale Management];

- 4. Click [+] at the top of the left status bar, enter the IP address and serial port number;

- 5. Click [Confirm] to connect.

- Update Firmware

- 1. Click [Select Firmware Zip] to upload the file(.zip);

- 2. Click [Download];

- 3. Select the way to update the firmware: Serial Port or Network(Wi-Fi, Ethernet).

- Note:

- Update via serial port , the scale will automatically restart;

- Update via Wi-Fi or Ethernet, the scale will not automatically restart. You need to manually restart the scale, and the firmware will update. (This method supports selecting multi-scales to update together.)

- PLU Edit

- 1. Create a new PLU: Click [+] to create a default data PLU.

- 2. Import PLU: Click [Import] to select the data sheet, and all PLU data in the sheet will be displayed.

- 3. Import PLU from the database: Click [Get Data From DB] will display all PLU data in the database.

- Note: The PLU in the database is not the PLU on the scale, but the database in the software backend. The software currently does not have the function to read PLU information from the scale.

- 4. Click the [Setting button] to select the PLU information that needs to be displayed.

- 5. If you need to modify PLU information, you can directly modify it in the corresponding cell. (Hovering over a cell will display the range of parameters that can be entered.)

- 6. Select the PLU to be download and click [Download].

- 7. Choose to clear all PLUs on the scale before downloading, or update the corresponding PLUs, and then click [Confirm].

- 8. Export in sheet format: Select the PLU to export, click [Export] and choose the folder to export.

- 9. Save to backend database: Select the PLU that needs to be saved and then click [Save to DB], choose to clear all PLUs in the backend database before saving, or update the corresponding PLUs, click [Confirm].

- 10. Get PLU Template: Click [Get PLU Template] and select the download path to obtain a form template that can be filled out with PLU.

- Weighing (Single)

- 1. Real time display of weighing data on the scale (supporting all units including PCS and PCT);

- 2. Click [TARE] and [ZERO] allows for weighing and zeroing operations on the scale;

- 3. Click the List button in the upper left corner to view the list of connected devices and make changes.

- Note：

- This function can be used by connecting to a serial port, Ethernet, or WiFi.

- Switching units is done on the scale.

- Weighing Data Collection (Single)

- 1. Click [Save] to manually save the current weighing data (with no quantity limit); Click [Export] to export all saved data (xlsx);

- 2. Click [Setting] and select Save Mode to Auto, it can be set to auto save mode;

- 3. You can also set the date format and delimiter for data export;

- 4. Enter the PLU number or name in the PLU Name column to call the PLU in the backend database; (The PLU here is not the PLU on the scale)

- 5. Click [Report Setting] to select the information displayed for weighing data; (Currently, there are no functional variables available: Product Code、Item Code、User Name、User NO）

- 6. Click the arrow in column of the report title row will sort the report in ascending/descending order.

- Note:

- 1. To save data, you need to clear the scale before saving the next transaction. Zero point will not be saved.

- 2. The upper and lower limit variables are the upper and lower limits set in the backend PLU, not the upper and lower limits set on the scale.

- 3. The saved data will not be cleared after closing/restarting the software.

- Check Weighing (Single)

- 1. Click [Edit] to set the upper and lower limits (the judgment is based on the upper and lower limits set in the background, not on the scale). After setting, place the weight and the corresponding alarm light will light up;

- 2. Click [Save] to save the current weighing record;

- 3. Click [Show Reports] to display the report;

- 4. Click [Setting] to set automatic saving. (Can be set to save when OK, HIGH, LOW, ALL.)

- 5. You can also set the date format and delimiter for data export;

- 6. Enter the PLU number or name in the [PLU Name] column to call the PLU in the backend database; (The PLU here is not the PLU on the scale)

- 7. Click [Report Setting] to select the information displayed for weighing data; (Currently, there are no functional variables available: Product Code, Item Code, User Name, User NO）

- 8. Click the arrow in a column of the report title row will sort the report in ascending/descending order.

- 9.Click [Export] to export all saved data (xlsx).

- Note:

- 1. The units for the upper and lower limits are the currently selected units on the scale. After setting the upper and lower limits in the software, the values of the upper and lower limits will not undergo unit conversion when switching units through the scale.

- 2. At present, the software determines the upper and lower limits from zero, while on the scale, it starts from 20d.

- Increment Weighing (Single)

- 1. Click [Start] to activate increment weighing function. (Only record the increment of the weighing value during loading.)

- 2. Click [Save] to save the current weighing record;

- 3. Click [Show Reports] to display the report;

- 4. Click [Setting] to set automatic saving.

- 5. You can also set the date format and delimiter for data export;

- 6. Enter the PLU number or name in the PLU Name column to call the PLU in the backend database; (The PLU here is not the PLU on the scale)

- 7. Click [Report Setting] to select the information displayed for weighing data; (Currently, there are no functional variables available: Product Code, Item Code, User Name, User NO）

- 8. Click the arrow in a column of the report title row will sort the report in ascending/descending order.

- 9.Click [Export] to export all saved data (xlsx).

- 10.Click [End] to stop the function.

- Note: The weight in the report refers to the increment rather than all the weight on the scale.

- Take Out Scale (Single)

- 1. The condition is that there must be weight on the scale. Click [Start] to activate take out scale function. (Only record the decrement of the weighing value during unloading.)

- 2. Click [Save] to save the current weighing record;

- 3. Click [Show Reports] to display the report;

- 4. Click [Setting] to set automatic saving.

- 5. In [Setting], you can also set the date format and delimiter for data export;

- 6. Enter the PLU number or name in the PLU Name column to call the PLU in the backend database; (The PLU here is not the PLU on the scale)

- 7. Click [Report Setting] to select the information displayed for weighing data; (Currently, there are no functional variables available: Product Code, Item Code, User Name, User NO）

- 8. Click the arrow in a column of the report title row will sort the report in ascending/descending order.

- 9.Click [Export] to export all saved data (xlsx).

- 10.Click [End] to stop the function.

- Note: The weight in the report refers to the decrement rather than all the weight on the scale.

- 多台秤管理

- 一、通过串口连接

- 1.通过串口线将设备与电脑连接；

- 2.输入正确的串口参数和信息，点击【连接】；

- 3.如果连接成功，左侧状态栏将显示设备在线。

- 5.点击【关闭串口】，断开串口连接；点击【打开串口】打开串口连接；

- 6.如果要更改秤名，请单击【重命名】，输入完后点击【确认】。

- 二、通过网口/Wi-Fi连接

- 1.点击左侧状态栏顶部的【+】按钮；

- 2.输入IPv4（IP地址）和Port（端口号）；

- 3.点击【确认】完成连接。

- 获取IP地址

- Wi-Fi：在主界面点击【Wi-Fi设置】，在左侧栏中选择要连接的Wi-Fi，输入密码，点击【连接】。如果连接成功，会显示IP地址。

- 网口：连接网线后，可以在秤的菜单中查看到IP地址。（菜单位置：F3-3 COM3 SHOW，其中F3-3 DHCP要设置为ON）

- 获取端口号

- Wi-Fi：进入秤的菜单F3-2 COM2 MODE，按TARE键三次查看端口号。

- 网口：进入秤的菜单F3-3 COM3 PORT，按一次TARE键查看端口号。

- 注：

- 如果要使用Wi-Fi连接T-Config，您需要先打开秤的Wi-Fi功能，并将运行T-Config的设备连接上该Wi-Fi；如果要使用网口连接T-Config，您需要先打开秤的网口功能，并将运行T-Config的设备连接到该网络。

- Wi-Fi设置

- 1.从左侧菜单栏中选择要连接的Wi-Fi；

- 2.输入密码，点击【连接】获取IP地址（IPv4）；

- 3.返回主界面，点击【多台秤管理】；

- 4.点击左侧的【+】，输入IP地址和串口号；

- 5.点击【确认】连接秤。

- 更新软件

- 1.点击【请选择更新文件Zip】并上传文件；

- 2.点击【下发】；

- 3. 选择固件更新方式：串口或网络（Wi-Fi、网口）；

- 注：

- 通过串口下发，秤会自动重启；

- 通过Wi-Fi或网口下发，可以批量选择多台秤，下发后秤不会自动重启，需要手动重启后才会更新软件。

- PLU编辑

- 1.创建新的PLU：点击【+】按钮创建默认数据的PLU。

- 2.导入PLU：点击【+】按钮，选择数据表上传，界面上会显示出所有PLU数据。

- 3.从数据库导入PLU：点击【导入】按钮。

- 注：数据库中的PLU指的不是秤上的PLU，而是T-Retail数据库的PLU。（目前软件暂不支持从秤上读取PLU。）

- 4.点击【设置】按钮 选择需要显示的PLU信息。

- 5.如果需要修改PLU信息，可以直接在相应的单元格中修改。（将鼠标悬停在单元格上将显示可以输入的参数范围。）

- 6.选择要下发的PLU，然后点击【下载】按钮。

- 7.此时界面会弹出确认框，您可以选择在下发前清除秤上的所有PLU，或是将对应的PLU进行更新。点击【确认】。

- 8.以表格格式导出：选择要导出的PLU，点击【导出】，然后选择要输出的文件夹。

- 9.保存到T-Retail数据库：选择需要保存的PLU，点击【保存数据库】，此时界面会弹出确认框，您可以选择在保存前清除后台的所有PLU，或是将对应的PLU进行更新。点击【确认】。

- 10.下载模板：点击【获取PLU模板】并选择下载路径，获取可以填写PLU的表格模板。

- 称重模式

- 1.此功能界面上会实时同步秤上的称重数据（支持所有单位包括PCS、PCT）；

- 2.可以点击【扣重】和【归零】对秤执行操作；

- 3.点击左上角的列表按钮可以在列表中查看已连接设备的并进行更换。

- 注：

- 1.此功能可以通过连接串口、网口或Wi-Fi使用。

- 2.T-Industry的称重单位与秤保持一致，但是切换单位需要在秤上执行。

- 重量收集模式

- 1.点击【保存】按钮，手动保存当前称重数据（无数量限制）；点击【导出】按钮，以表格形式导出保存的所有数据；

- 2.点击【设置】，可以将【保存模式】切换为【自动】；

- 3.在【设置】中，您还可以设置数据导出的日期格式和数据分隔符；

- 4.在【PLU名字】中输入PLU编号或名称，以调用T-Industry数据库中的PLU（此处的PLU不是秤上的PLU）；

- 5.点击【报表设置】，可以选择称重数据显示的信息；（目前暂无功能的变量：Product Code、Item Code、User Name、User NO）

- 6.点击标题行中的上下箭头，可以对报表进行升/降的排序。

- 注：

- 1.需要先清空秤盘，才会保存下一条称重的数据（零点不会被保存）。

- 2.【报表设置】中的上下限是T-Industry数据中PLU设置的上下限，并非秤上设置的上下限。

- 3.关闭/重启软件，已保存的数据不会被清除。

- 检重秤模式

- 1.点击【编辑】设置上下限（判断是基于软件设置的上下限，而不是秤）。设置完成后，放置重量，相应的报警灯会亮起；

- 2.点击【保存】按钮，保存当前称重记录；

- 3.点击【显示报表】，会在界面下方显示已保存的称重记录；

- 4.点击【设置】可以切换为自动保存模式，并可以设置为在OK/HIGH/LOW/ALL时保存。

- 5.在【PLU名字】中输入PLU编号或名称，以调用T-Industry数据库中的PLU（此处的PLU不是秤上的PLU）；

- 6.点击【报表设置】，可以选择称重数据显示的信息；（目前暂无功能的变量：Product Code、Item Code、User Name、User NO）

- 7.点击标题行中的上下箭头，可以对报表进行升/降的排序；

- 8.点击【导出】按钮，以表格形式导出保存的所有数据；

- 注：

- 1.上下限的单位是当前秤上选择的单位。在软件中设置好上下限后，并通过秤切换单位后，上下限的值不会进行单位换算。

- 2.目前软件判断上下限是从大于零点开始（一般秤上是从20d开始）。

- 加法秤模式

- 1.点击【开始】以启动加法秤功能，记录加载过程中称重值的增量。

- 2.点击【显示报表】，会在界面下方显示已保存的记录；

- 3.点击【设置】，可以将【保存模式】切换为【自动】；

- 4.在【设置】中，您还可以设置数据导出的日期格式和数据分隔符；

- 5.在【PLU名字】中输入PLU编号或名称，以调用T-Industry数据库中的PLU（此处的PLU不是秤上的PLU）；

- 6.点击【报表设置】，可以选择称重数据显示的信息；（目前暂无功能的变量：Product Code、Item Code、User Name、User NO）

- 7.点击标题行中的上下箭头，可以对报表进行升/降的排序；

- 8.点击【导出】按钮，以表格形式导出保存的所有数据；
9.点击【停止】按钮，可以关闭此功能。

- 注：报表中的重量是指增量，而不是秤盘上的所有重量。

- 减法秤模式

- 1.点击【开始】以启动减法秤功能，记录卸载过程中称重值的减量（条件是秤上必须有重量）。

- 2.点击【显示报表】，会在界面下方显示已保存的记录；

- 3.点击【设置】，可以将【保存模式】切换为【自动】；

- 4.在【设置】中，您还可以设置数据导出的日期格式和数据分隔符；

- 5.在【PLU名字】中输入PLU编号或名称，以调用T-Industry数据库中的PLU（此处的PLU不是秤上的PLU）；

- 6.点击【报表设置】，可以选择称重数据显示的信息；（目前暂无功能的变量：Product Code、Item Code、User Name、User NO）

- 7.点击标题行中的上下箭头，可以对报表进行升/降的排序；

- 8.点击【导出】按钮，以表格形式导出保存的所有数据；
9.点击【停止】按钮，可以关闭此功能。

- 注：报表中的重量是指减量，而不是秤盘上的所有重量。