T-Retail功能操作说明20250113

- Multi-scale Management

- Connect via serial port

- 1. Connect the scale to the PC device with a serial port cable;

- 2. Enter the correct serial port information;

- 3. Click [Connect];

- 4. If the connection is successful, the left status bar will display Online.

- 5. Click [Close Port] to disconnect the serial port connection; Click [Open Port] to open the serial port connection;

- 6. If you want to change the scale name, click [Rename] to enter it and click [Confirm].

- Connect via Ethernet/Wi-Fi

- 1. Click [+] at the top of the left status bar;

- 2. Enter the IPv4 (IP Address) and Port (Port Number);

- 3. Click [Confirm] to complete the connection.

- (1) Get IP Address

- Wi-Fi: Click [Wi-Fi Setting] on the main interface, select the Wi-Fi you want to connect to in the left column, enter the password and click [Connect]. If the connection is successful, the IP Address will be displayed.

- Ethernet: After connecting the network cable, enter the menu F3-3 COM3 SHOW on the scale (F3-3 DHCP set to ON) to get the IP Address.

- (2) Get Port Number

- Wi-Fi: Enter the menu F3-2 COM2 MODE on the scale, press the TARE key three times to view the port number.

- Ethernet: Enter the menu F3-3 COM3 PORT on the scale, press the TARE key once to view the port number.

- Note:

- To use Wi-Fi connection, you need to turn on the scale's Wi-Fi function and connect the device running the software to the target Wi-Fi as well.

- To use Ethernet connection, you need to turn on the network port function of the scale and connect the device running the software to the target Wi-Fi as well.

- Wi-Fi Setting

- 1. Select the Wi-Fi you want to connect to from the left menu bar;

- 2. Enter the password and click [Connect] to obtain the IP address;

- 3. Return to the main interface and enter [Multi-scale Management];

- 4. Click [+] at the top of the left status bar, enter the IP address and serial port number;

- 5. Click [Confirm] to connect.

- Update Firmware

- 1. Click [Select Firmware Zip] to upload the file(.zip);

- 2. Click [Download];

- 3. Select the way to update the firmware: Serial Port or Network(Wi-Fi, Ethernet).

- Note:

- Update via serial port , the scale will automatically restart;

- Update via Wi-Fi or Ethernet, the scale will not automatically restart. You need to manually restart the scale, and the firmware will update. (This method supports selecting multi-scales to update together.)

- Receipt Format Download

- 1. Follow the prompts to select the corresponding print format for uploading;

- 2. Click [Download] to download the format(.fmt);

- 3. If multiple networks are connected, you need to select the corresponding network before downloading.

- Receipt Design

- 1. Select printer protocol, length and width of the ticket, printing direction and other information;

- 2. When editing the print format, there are 4 variables to choose from in the left menu bar;

- Free Text: Text variables that can be freely inputted

- Dividing Line: A horizontal line used for segmentation (after selecting this variable twice, the content in the middle of the variable can appear repeatedly, such as the accumulated content each time)

- Variable: Variables required for weighing scales

- Price Variable: Variables required for the price computing scale

- 3. After editing the format, click [Save Format] to save the print format.

- Note: The weighing scale and pricing scale cannot use the wrong variable, otherwise the printed output will be empty.

- PLU Edit

- 1. Create a new PLU: Click [+] to create a default data PLU.

- 2. Import PLU: Click [Import] to select the data sheet, and all PLU data in the sheet will be displayed.

- 3. Import PLU from the database: Click [Get Data From DB] will display all PLU data in the database.

- Note: The PLU in the database is not the PLU on the scale, but the database in the software backend. The software currently does not have the function to read PLU information from the scale.

- 4. Click the [Setting button] to select the PLU information that needs to be displayed.

- 5. If you need to modify PLU information, you can directly modify it in the corresponding cell. (Hovering over a cell will display the range of parameters that can be entered.)

- 6. Select the PLU to be download and click [Download].

- 7. Choose to clear all PLUs on the scale before downloading, or update the corresponding PLUs, and then click [Confirm].

- 8. Export in sheet format: Select the PLU to export, click [Export] and choose the folder to export.

- 9. Save to backend database: Select the PLU that needs to be saved and then click [Save to DB], choose to clear all PLUs in the backend database before saving, or update the corresponding PLUs, click [Confirm].

- 10. Get PLU Template: Click [Get PLU Template] and select the download path to obtain a form template that can be filled out with PLU.

- Variable Value Setting

- 1. Edit header, footer, and operator information;

- 2. Click [Download] to send to the scale.

- Retail Detail Report

- 1. This function interface can display the data printed after each checkout;

- If the service is not installed, you need to click [Service] on the right and click [Install Service]; When prompted that the service has stopped, click [Start Service] and it will prompt that it is running. Finally, click [Confirm] to complete the process;

- 2. After the balance is printed, the printing content of each order will be displayed on this interface, click [Refresh List] and you can get the latest records;

- 3. Click [Export] and you can download the report.

- Note: This function can be used via Ethernet / WiFi, and cannot be used via serial port.

- 多台秤管理

- 一、通过串口连接

- 1.通过串口线将设备与电脑连接；

- 2.输入正确的串口参数和信息，点击【连接】；

- 3.如果连接成功，左侧状态栏将显示设备在线。

- 5.点击【关闭串口】，断开串口连接；点击【打开串口】打开串口连接；

- 6.如果要更改秤名，请单击【重命名】，输入完后点击【确认】。

- 二、通过网口/Wi-Fi连接

- 1.点击左侧状态栏顶部的【+】按钮；

- 2.输入IPv4（IP地址）和Port（端口号）；

- 3.点击【确认】完成连接。

- 获取IP地址

- Wi-Fi：在主界面点击【Wi-Fi设置】，在左侧栏中选择要连接的Wi-Fi，输入密码，点击【连接】。如果连接成功，会显示IP地址。

- 网口：连接网线后，可以在秤的菜单中查看到IP地址。（菜单位置：F3-3 COM3 SHOW，其中F3-3 DHCP要设置为ON）

- 获取端口号

- Wi-Fi：进入秤的菜单F3-2 COM2 MODE，按TARE键三次查看端口号。

- 网口：进入秤的菜单F3-3 COM3 PORT，按一次TARE键查看端口号。

- 注：

- 如果要使用Wi-Fi连接T-Config，您需要先打开秤的Wi-Fi功能，并将运行T-Config的设备连接上该Wi-Fi；如果要使用网口连接T-Config，您需要先打开秤的网口功能，并将运行T-Config的设备连接到该网络。

- Wi-Fi设置

- 1.从左侧菜单栏中选择要连接的Wi-Fi；

- 2.输入密码，点击【连接】获取IP地址（IPv4）；

- 3.返回主界面，点击【多台秤管理】；

- 4.点击左侧的【+】按钮，输入IP地址和串口号；

- 5.点击【确认】连接秤。

- 更新软件

- 1.点击【请选择更新文件Zip】并上传文件；

- 2.点击【下发】；

- 3. 选择固件更新方式：串口或网络（Wi-Fi、网口）；

- 注：

- 通过串口下发，秤会自动重启；

- 通过Wi-Fi或网口下发，可以批量选择多台秤，下发后秤不会自动重启，需要手动重启后才会更新软件。

- 下发票据打印格式

- 1.根据提示选择相应的票据打印格式并上传；

- 2.点击【下发】；

- 3.如果连接了多个网络，则需要在下发前选择设备对应的网络。

- 票据设计

- 1.选择打印机协议、票据长度/宽度、打印方向等信息；

- 2.编辑打印格式时，左侧菜单栏中有4个变量可供选择；

- 自由文本：可以自由输入的文本变量

- 分割线：用于分割的水平线（选择该变量两次后，变量中间的内容可以重复出现，比如每次累积的内容）

- 变量：计重秤所需的变量

- 价格变量：计价秤所需的变量

- 3.编辑好票据打印格式后，点击【保存格式】。

- 注：计重秤和计价秤不能使用错误的变量，否则打印出来是空的。

- PLU编辑

- 1.创建新的PLU：点击【+】按钮创建默认数据的PLU。

- 2.导入PLU：点击【+】按钮，选择数据表上传，界面上会显示出所有PLU数据。

- 3.从数据库导入PLU：点击【导入】按钮。

- 注：数据库中的PLU指的不是秤上的PLU，而是T-Retail数据库的PLU。（目前软件暂不支持从秤上读取PLU。）

- 4.点击【设置】按钮 选择需要显示的PLU信息。

- 5.如果需要修改PLU信息，可以直接在相应的单元格中修改。（将鼠标悬停在单元格上将显示可以输入的参数范围。）

- 6.选择要下发的PLU，然后点击【下载】按钮。

- 7.此时界面会弹出确认框，您可以选择在下发前清除秤上的所有PLU，或是将对应的PLU进行更新。点击【确认】。

- 8.以表格格式导出：选择要导出的PLU，点击【导出】，然后选择要输出的文件夹。

- 9.保存到T-Retail数据库：选择需要保存的PLU，点击【保存数据库】，此时界面会弹出确认框，您可以选择在保存前清除后台的所有PLU，或是将对应的PLU进行更新。点击【确认】。

- 10.下载模板：点击【获取PLU模板】并选择下载路径，获取可以填写PLU的表格模板。

- 变量设置

- 1.编辑页眉、页脚、操作员信息；

- 2.点击【下发】按钮下发至秤。

- 零售详单

- 1.此功能界面可以显示每次结账打印后的数据；

- 注：如果未安装服务，您需要点击右侧的【服务】-【安装服务】，当系统提示服务已停止时，点击【开始服务】，系统会提示服务正在运行。最后，点击【确认】完成服务的安装；

- 2.结账打印后，每笔订单的数据都将显示在此界面上，点击【刷新列表】可以获取最新的订单记录；

- 3.点击【导出】，可以将报表下载至本地。

- 注意：此功能可以通过网口/Wi-Fi连接使用，不能通过串口使用。